XJEL2665 Unit 4 Assessment

2024-25

Design Validation Report

Two-mode Timer

201691164

Xinzhe Liu

2025.05.08

Statement of Academic Integrity

By submitting work, I confirm that I have read and understood the University’s definition of Academic Integrity and the University’s rules on Academic Misconduct and am aware that if I commit an academic misconduct offence there can be serious disciplinary consequences.

I am aware that the University defines plagiarism as presenting someone else’s work, in whole or in part, as your own. Work means any intellectual output, and typically includes text, data, images, code, sound or performance.

I promise that in the attached submission I have not presented anyone else’s work, in whole or in part, as my own and I have not colluded with others in the preparation of this work. Where I have taken advantage of the work of others, I have given full acknowledgement. I have not resubmitted my own work or part thereof without specific written permission to do so from the University staff concerned when any of this work has been or is being submitted for marks or credits even if in a different module or for a different qualification or completed prior to entry to the University. I have read and understood the University’s published rules on plagiarism and also any more detailed rules specified at the school or module level. I know that if I commit plagiarism I can be expelled from the University and that it is my responsibility to be aware of the University’s regulations on plagiarism and their importance.

I re-confirm my consent to the University copying and distributing any or all of my work in any form and using third parties (who may be based outside the EU/EEA) to monitor breaches of regulations, to verify whether my work contains plagiarised material, and for quality assurance purposes.

I confirm that I have declared all mitigating circumstances that may be relevant to the assessment of this piece of work and that I wish to have taken into account. I am aware of the University’s policy on mitigation and the School’s procedures for the submission of statements and evidence of mitigation. I am aware of the penalties imposed for the late submission of coursework.

Section A: Submodules Validation

A1 Clock Divider A1.1 Overview of the Testing Process Reset the signal low for 100ns, then start the 1Hz counter and the 100Hz counter. After each counter counts twice, end the simulation. A1.2 Test procedures - The reset port rst_n is pulled low for 100 ns and then released. - The 100 Hz counter and the 1 Hz counter start counting respectively. - The simulation ends when the 100 Hz output counts twice and the 1 Hz output counts twice (since the trigger time of 1 Hz is much longer than that of 100 Hz, the actual counting time is based on the 1 Hz counter). A1.3 Test result - The initial reset is pulled low for 100 ns. At 100 ns, the reset changes from 0 to 1. - The 100 Hz counter flips once every 5 ms, meaning it counts once every 10 ms. - The 1 Hz counter flips once every 500 ms, meaning it counts once every 1000 ms. - When the 1 Hz counter is about to count for the second time (since the rising edge has not been triggered, the 1 Hz counter has counted a total of 1.5 cycles but still shows 1 count), the 100 Hz counter has counted 150 times. A2 Program Counter (PC) A2.1 Overview of the Testing Process Raise the reset to verify the asynchronous reset. Set load verification synchronous loading Set inc verification for synchronous auto-increment Simultaneously verify the load priority at high level Print the result and stop the simulation. A2.2 Test procedures - Set the initial value: ResetVal = 8'hA5; LoadVal = 8'd5; reset = 0; load = 0; inc = 0; - When t = 10 ns , reset test: reset = 1 ( 1 ns ), reset = 0. - When t = 25 ns , load test: load = 1 ( 10 ns ), load = 0. - When t = 35 ns , increment test: inc = 1 ( 20 ns ), inc = 0. - When t = 65 ns , priority test: load = 1 ( 10 ns ), inc = 1 ( 10 ns ). A2.3 Test result t = 0 ns (Rising edge of clk), reset = 0 , load = 0 , increment = 0 , output = None; t = 10 ns (Rising edge of clk), reset = 1 , load = 0 , increment = 0 , output = ResetVal = 0xA5 = 165; t = 25 ns (Rising edge of clk), reset = 0 , load = 1 , increment = 0 , output = LoadVal = 10; t = 35 ns (Rising edge of clk), reset = 0 , load = 0 , increment = 1 , output = 10 + 1 = 11; t = 45 ns (Rising edge of clk), reset = 0 , load = 0 , increment = 1 , output = 11 + 1 = 12; t = 55 ns (Rising edge of clk), reset = 0 , load = 0 , increment = 0 , output = 12; t = 65 ns (Rising edge of clk), reset = 0 , load = 1 , increment = 1 , output = LoadVal = 7; (Load priority) A3 Binary to BCD A3.1 Overview of the Testing Process Iterate through all values from 0 to 99 and check if the units and tens digits are correct. A3.2 Test procedures Test all values of i from 0 to 99. Check if the tens digit is equal to i/10 and the units digit is equal to i%10. If correct, print 'pass'; if incorrect, print 'should be (correct result)'. A3.3 Test result All results passed. A4 BCD to 7-Segment A4.1 Overview of the Testing Process Iterate through all values from 0 to 9 and check if the 7-segment display encoding is correct. A4.2 Test procedures Test all values from 0 to 9 to determine if the encoding of the digital tube is consistent with the mapping table. A4.3 Test result The result is correct. A5 Seven-Segment Encoder A5.1 Overview of the Testing Process Output the seven-segment digital tube time code in two modes respectively. A5.2 Test procedures - ModeSel = 0 (stopwatch): 0:0, 34:12, 99:99; - ModeSel = 1 (count down): 0:0 corresponds to display 2:00, 0:5 corresponds to display 1:55, 2:0 corresponds to display 0:0. A5.3 Test result A6 Timer Core Logic A6.1 Overview of the Testing Process Driven by a 100 MHz clock, first test the reset, then test the low-active and high-active bit counting and carry, pause and restart the counting. A6.2 Test procedures - To observe the wraparound behavior within a limited time, set the maximum value MAX_VAL_LSB to 5 and MAX_VAL_MSB to 3. In the main code, it can be set to 99. - t = 0 ns, asynchronous reset, the counter is initialized to RESET_VAL. - t = 35 ns, StartStop = 1, LSB + 1 - t = 85 ns, LSB reaches 5, it wraps around to 0 and generates a carry, and MSB +1. - t = 145 ns, LSB → MSB +1 - t = 205 ns, the third time LSB→ MSB +1 - t = 230~260 ns, StartStop=0 → all remain unchanged (on hold) - t = 265 ns, StartStop=1 ，LSB from 2 → 3 - t = 295 ns, LSB → carry out; MSB reach 3 → 0 A6.3 Test result

Section B: Two-mode Timer (Top-level module) Validation

B Main code B Overview of the Testing Process Verify whether the seven-segment code decoding output is correct under both Stopwatch (100 Hz) and Timer (1 Hz) modes respectively. In each mode, check whether the counting can start and pause by pulling up/pulling down StartStop. B Test procedures - After reset ModeSel=0; StartStop=1, wait ≥20 ms（At least two 100 Hz cycles. - StartStop=0 stop, Output freeze. - Reset again, ModeSel = 1; StartStop = 1 to start the countdown, wait for ≥ 2 seconds (at least 2 cycles of 1 Hz); StartStop = 0 to pause and freeze the output. - Test finish. B Test result

Appendix

Please attach ALL your Quartus codes here.

Note: please make sure you submit the code in pure text format, not image! Fail to do so will severely affect your report mark!

module MainCode (

input  wire        		CLK_50MHz,    // 50MHz clk

input  wire        		rst_n,        // reset(effective low)

input  wire        		StartStop,    // start/stop

input  wire        		ModeSel,      // mode select：0 Stopwatch（100Hz），1 count down（1Hz）

output wire	[6:0]  	HexMSBH,      // MSB 7-seg

output wire	[6:0]  	HexMSBL,      // MSB 7-seg

output wire	[6:0]		HexLSBH,      // LSB 7-seg

output wire	[6:0]  	HexLSBL,      // LSB 7-seg

output wire				DOT           // dot control

);

// clock divider：50MHz → 100Hz / 1Hz

wire					 clk100Hz, clk1Hz;

ClockDivider u_clkdiv (

.CLK_50MHz 	 (CLK_50MHz),

.rst_n     	 (rst_n),

.CLK_100Hz 	 (clk100Hz),

.CLK_1Hz   	 (clk1Hz)

);

// mode-driven clock selection: 100Hz for stopwatch, 1Hz for timer.

wire					 select_clk;

assign 				 select_clk = ModeSel ? clk1Hz : clk100Hz;

// parameter (ModeSel = 0 → Stopwatch; = 1 → Timer)

wire	[7:0]			 ResetValLSB = 8'd0;

wire	[7:0]			 MaxValLSB   = 8'd99;

wire	[7:0] 		 ResetValMSB = 8'd0;

wire	[7:0] 		 MaxValMSB   = 8'd99;

// core counting logic

wire [7:0] LSBbinaryout, MSBbinaryout;

TimerCoreLogic u_timer (

.clk          (select_clk),

.rst_n        (rst_n),

.StartStop    (StartStop),

.ResetValLSB  (ResetValLSB),

.MaxValLSB    (MaxValLSB),

.ResetValMSB  (ResetValMSB),

.MaxValMSB    (MaxValMSB),

.LSBbinaryout (LSBbinaryout),

.MSBbinaryout (MSBbinaryout)

);

// Binary to Seven-Segment Code (Internal Reverser for Up and Down Counting)

SevenSegEncoder u_seg (

.LSBBinary 	 (LSBbinaryout),

.MSBBinary 	 (MSBbinaryout),

.ModeSel   	 (ModeSel),

.HexMSBH   	 (HexMSBH),

.HexMSBL   	 (HexMSBL),

.HexLSBH   	 (HexLSBH),

.HexLSBL   	 (HexLSBL)

);

// Decimal point blinking: Blinking at the same frequency as the selected clock

assign DOT = select_clk;

endmodule

`timescale 1ns/1ps

module MainCode_tb;

reg  				CLK_50MHz = 0;

reg  				rst_n      = 0;

reg  				StartStop  = 0;

reg  				ModeSel    = 0;

wire [6:0] 		HexMSBH, HexMSBL, HexLSBH, HexLSBL;

wire       		DOT;

MainCode uut (

.CLK_50MHz 	(CLK_50MHz),

.rst_n      	(rst_n),

.StartStop  	(StartStop),

.ModeSel    	(ModeSel),

.HexMSBH    	(HexMSBH),

.HexMSBL    	(HexMSBL),

.HexLSBH    	(HexLSBH),

.HexLSBL    	(HexLSBL),

.DOT        	(DOT)

);

// system clk 50MHz, period 20ns

always #10 CLK_50MHz = ~CLK_50MHz;

// show message

initial begin

$monitor("T=%0t | ModeSel=%b rst_n=%b StartStop=%b | MSBH=%h MSBL=%h LSBH=%h LSBL=%h DOT=%b",

$time, ModeSel, rst_n, StartStop, HexMSBH, HexMSBL, HexLSBH, HexLSBL, DOT);

end

initial begin

// reset

rst_n = 0; #100; rst_n = 1; #100;

// mode A

ModeSel   = 0;

StartStop = 1;

// wait 20 ms (100 Hz output for 2 cycles)

#20000000;

StartStop = 0;

// mode B

rst_n     = 0; #100; rst_n = 1; #100;

ModeSel   = 1;

StartStop = 1;

// wait 2 s (2 cycles output at 1 Hz)

#2000000000;

StartStop = 0;

$stop;

end

endmodule

module ClockDivider (

input wire 		CLK_50MHz, 		//50MHz clock input

input wire 		rst_n, 			//Active-LOW reset signal

output reg 		CLK_100Hz,	 	//100Hz clock; You can tweak the output type accordingly.

output reg 		CLK_1Hz			//1Hz clock; You can tweak the output type accordingly.

);

// Frequency divider counter

reg [17:0] 		count_100;

reg [25:0] 		count_1;

// main logic: Reset triggered by the rising edge of the clock

always @(posedge CLK_50MHz or negedge rst_n) begin

if (!rst_n) begin

// reset

count_100    <= 18'd0;

CLK_100Hz   	<= 1'b0;

count_1      <= 26'd0;

CLK_1Hz     	<= 1'b0;

end else begin

// At 100 Hz frequency division, the counter flips when it reaches 250000 - 1

if (count_100 == 18'd250_000 - 1) begin

count_100   <= 18'd0;

CLK_100Hz   <= ~CLK_100Hz;

// If not full, continue counting

end else begin

count_100   <= count_100 + 1;

end

// At 1 Hz frequency division, the counter flips when it reaches 25000000 - 1

if (count_1 == 26'd25_000_000 - 1) begin

count_1     <= 26'd0;

CLK_1Hz     <= ~CLK_1Hz;

end else begin

count_1     <= count_1 + 1;

end

end

end

endmodule

`timescale 1ns/1ps

module ClockDivider_tb;

reg 		CLK_50MHz;    // 50 MHz clk

reg 		rst_n;        // reset (effective low)

wire 	CLK_100Hz;   // 100 Hz clk

wire 	CLK_1Hz;     // 1 Hz clk

ClockDivider uut (

.CLK_50MHz	(CLK_50MHz),

.rst_n			(rst_n),

.CLK_100Hz	(CLK_100Hz),

.CLK_1Hz		(CLK_1Hz)

);

// 50 MHz clock generation: period 20 ns, rising edge interval 10 ns

initial CLK_50MHz = 0;

always #10 CLK_50MHz = ~CLK_50MHz;

// reset

initial begin

rst_n = 0;

#100;

rst_n = 1;

end

// rising edge counting

integer count100 = 0;

integer count1   = 0;

// 100 Hz

always @(posedge CLK_100Hz) begin

count100 = count100 + 1;

$display("Time=%0t: %0d times CLK_100Hz rising edge", $time, count100);

end

// 1 Hz

always @(posedge CLK_1Hz) begin

count1 = count1 + 1;

$display("Time=%0t:  %0d times CLK_1Hz rising edge", $time, count1);

end

// process ends when the 100 Hz output occurs twice and the 1 Hz output occurs twice.

initial begin

wait(count100 == 2);

wait(count1   == 2);

$display("test finish: 100 Hz change 2 times，1 Hz change 2 times，finish time %0t", $time);

$stop;

end

endmodule

module TimerCoreLogic #(

// reset value of the counter and the wraparound value when the value reaches the upper limit

parameter 		RESET_VAL_LSB = 8'd0,

parameter 		MAX_VAL_LSB   = 8'd99,

parameter 		RESET_VAL_MSB = 8'd0,

parameter 		MAX_VAL_MSB   = 8'd99

)(

input         clk,            // 100Hz or 1Hz clk

input         rst_n,          // reset(effective low)

input         StartStop,      // start/stop

input  [7:0]  ResetValLSB,

input  [7:0]  MaxValLSB,

input  [7:0]  ResetValMSB,

input  [7:0]  MaxValMSB,

output [7:0]  LSBbinaryout,   // LSB counter

output [7:0]  MSBbinaryout    // MSB counter

);

wire	  [7:0] 		count_lsb;

wire   [7:0] 		count_msb;

wire       		carry_lsb = (count_lsb == MaxValLSB) & StartStop;	// overflow control: when lower bit reaches the upper limit and is running, a carry pulse is generated

wire       		carry_msb = (count_msb == MaxValMSB) & carry_lsb; // Overflow control: when higher bit reaches the upper limit and the low bit also generates a carry, a high-bit wraparound pulse is produced.

// output assignment

assign	LSBbinaryout = count_lsb;

assign	MSBbinaryout = count_msb;

// low-bit counter: rising edge driven

ProgramCounter pc_lsb (

.clk      (clk),

.ResetVal (ResetValLSB),

.LoadVal  (ResetValLSB),

.reset    (~rst_n),

.load     (carry_lsb),

.inc      (StartStop),

.PCoutput (count_lsb)

);

// high-order counter: carry from the lower order driven

ProgramCounter pc_msb (

.clk      (clk),

.ResetVal (ResetValMSB),

.LoadVal  (ResetValMSB),

.reset    (~rst_n),

.load     (carry_msb),

.inc      (carry_lsb),

.PCoutput (count_msb)

);

endmodule

`timescale 1ns/1ps

module TimerCoreLogic_tb;

reg         clk;

reg         rst_n;

reg         StartStop;

reg 	[7:0]  ResetValLSB, MaxValLSB;

reg 	[7:0]  ResetValMSB, MaxValMSB;

wire [7:0]  LSBbinaryout;

wire [7:0]  MSBbinaryout;

TimerCoreLogic #(

.RESET_VAL_LSB	(8'd0),

.MAX_VAL_LSB  	(8'd5),

.RESET_VAL_MSB	(8'd0),

.MAX_VAL_MSB  	(8'd3)

) uut (

.clk         	(clk),

.rst_n       	(rst_n),

.StartStop   	(StartStop),

.ResetValLSB 	(ResetValLSB),

.MaxValLSB   	(MaxValLSB),

.ResetValMSB 	(ResetValMSB),

.MaxValMSB   	(MaxValMSB),

.LSBbinaryout	(LSBbinaryout),

.MSBbinaryout	(MSBbinaryout)

);

// system clk

initial 	clk = 1'b0;

always #5 	clk = ~clk;

// reset, start/stop

initial begin

// initial value

ResetValLSB 	= 8'd0;

MaxValLSB 	= 8'd5;		// reaches 5, overflows by 1

ResetValMSB 	= 8'd0;

MaxValMSB 	= 8'd3;		// reaches 3, overflows by 1

rst_n       	= 1'b0;

StartStop   	= 1'b0;

#20;

// release and reset

rst_n = 1'b1;

#10;

// start

StartStop = 1'b1;

#200;

// stop

StartStop = 1'b0;

#30;

// restart

StartStop = 1'b1;

#2000;

$display("test finish");

$stop;

end

initial begin

$display(" time(ns) | LSB | MSB ");

$monitor(" %8t | %3d | %3d", $time, LSBbinaryout, MSBbinaryout);

end

endmodule

module ProgramCounter (

input					clk,					//Clock Signal (100Hz or 1Hz)

input 	[7:0]		ResetVal,			//input value to update the PC wehn load is active

input 	[7:0]		LoadVal,				//input value to update the PC when reset is active

input					reset,				//An active-high singal to rest the PC (exectute without clk)

input					load,					//An active-high signal to load value in PC (function activated with clk)

input					inc,					//An active-high singal to start the counting

output reg [7:0] 	PCoutput				//PC output signal in binary

);

// asynchronous reset + synchronous load/increment/hold

always @(posedge clk or posedge reset) begin

if (reset) begin

PCoutput <= ResetVal;            // asynchronous reset first

end

else if (load) begin

PCoutput <= LoadVal;             // synchronous load

end

else if (inc) begin

PCoutput <= PCoutput + 1;        // increment

end

else begin

PCoutput <= PCoutput;            // hold

end

end

endmodule

`timescale 1ns/1ps

module ProgramCounter_tb;

reg         	clk;

reg         	reset;      // asynchronous high-level reset

reg         	load;       // synchronous loading

reg         	inc;        // synchronous auto-increment

reg  [7:0]  	ResetVal;   // value loaded during reset

reg  [7:0]  	LoadVal;    // value loaded during loading

wire [7:0]  	PCoutput;   // counter output

ProgramCounter dut (

.clk      (clk),

.ResetVal (ResetVal),

.LoadVal  (LoadVal),

.reset    (reset),

.load     (load),

.inc      (inc),

.PCoutput (PCoutput)

);

// clk：period 10ns（100MHz）

initial clk = 0;

always #5 clk = ~clk;

initial begin

// initial value

ResetVal = 8'hA5;

LoadVal  = 8'd5;

reset    = 0;

load     = 0;

inc      = 0;

#10;

// asynchronous reset test

reset 	  = 1;

#1;

$display("after reset PCoutput = 0x%h (expectation = 0xA5)", PCoutput);

reset 	  = 0;

#10;

// sStep loading test

LoadVal  = 8'd10;

load     = 1;

#10;

load     = 0;

$display("after load PCoutput = %0d (expectation = 10)", PCoutput);

#10;

// synchronous auto-increment test

inc      = 1;

#10; $display("1'st auto-increment PCoutput = %0d (expectation = 11)", PCoutput);

#10; $display("2'st auto-increment PCoutput = %0d (expectation = 12)", PCoutput);

inc      = 0;

#10; $display("stop auto-increment PCoutput = %0d (expectation maintain 12)", PCoutput);

// priority test: When both load and inc are valid simultaneously, load takes priority.

LoadVal  = 8'd7;

load     = 1;

inc      = 1;

#10;

$display("when load and inc =1 , PCoutput = %0d (expectation = 7)", PCoutput);

load 	  = 0;

inc  	  = 0;

#10;

$display("test finish");

$stop;

end

endmodule

module SevenSegEncoder (

input  [7:0]	LSBBinary,			//The LSB binary value input

input  [7:0]	MSBBinary,			//The MSB bianry value input

input 			ModeSel,				//Control signal for the interal Reverser

output [6:0] 	HexMSBH,				//The 7-Seg display Signal for higer-digit in MSB

output [6:0] 	HexMSBL,				//The 7-Seg display Signal for lower-digit in MSB

output [6:0] 	HexLSBH,				//The 7-Seg display Signal for higer-digit in LSB

output [6:0] 	HexLSBL				//The 7-Seg display Signal for lower-digit in LSB

);

// 2-minute countdown: min

wire	[7:0] 	show_bin_min = ModeSel

? (8'd2 - MSBBinary - (LSBBinary != 8'd0 ? 8'd1 : 8'd0))

: MSBBinary;

// 2-minute countdown: sec

wire [7:0] 	show_bin_sec = ModeSel

? ((LSBBinary == 8'd0) ? 8'd0 : 8'd60 - LSBBinary)

: LSBBinary;

// binary to BCD

wire [3:0] 	bcd_min_tens, bcd_min_ones;

wire [3:0] 	bcd_sec_tens, bcd_sec_ones;

bin2bcd u_b2b_min (

.bin  (show_bin_min),

.tens (bcd_min_tens),

.ones (bcd_min_ones)

);

bin2bcd u_b2b_sec (

.bin  (show_bin_sec),

.tens (bcd_sec_tens),

.ones (bcd_sec_ones)

);

// BCD counvert to 7-seg

bcd_to_7seg u_seg_min_h (.bcd(bcd_min_tens), .seg(HexMSBH));

bcd_to_7seg u_seg_min_l (.bcd(bcd_min_ones), .seg(HexMSBL));

bcd_to_7seg u_seg_sec_h (.bcd(bcd_sec_tens), .seg(HexLSBH));

bcd_to_7seg u_seg_sec_l (.bcd(bcd_sec_ones), .seg(HexLSBL));

endmodule

`timescale 1ns/1ps

module SevenSegEncoder_tb;

reg  [7:0] 	LSBBinary;

reg  [7:0] 	MSBBinary;

reg        	ModeSel;

wire [6:0] 	HexMSBH, HexMSBL, HexLSBH, HexLSBL;

SevenSegEncoder uut (

.LSBBinary (LSBBinary),

.MSBBinary (MSBBinary),

.ModeSel   (ModeSel),

.HexMSBH   (HexMSBH),

.HexMSBL   (HexMSBL),

.HexLSBH   (HexLSBH),

.HexLSBL   (HexLSBL)

);

initial begin

$display("Time   Mode MSB LSB | MSBH    MSBL    LSBH    LSBL");

// mode A (ModeSel=0): show MSBBinary.LSBBinary

ModeSel   = 0;

MSBBinary = 8'd0;   LSBBinary = 8'd0;   #20;				// 00.00, mode = 0

$display("%4dns     %b    %2d   %2d | %b %b  %b %b",

$time, ModeSel, MSBBinary, LSBBinary,

HexMSBH, HexMSBL, HexLSBH, HexLSBL);

MSBBinary = 8'd34;  LSBBinary = 8'd12;  #20;				// 34.12, mode = 0

$display("%4dns     %b    %2d   %2d | %b %b  %b %b",

$time, ModeSel, MSBBinary, LSBBinary,

HexMSBH, HexMSBL, HexLSBH, HexLSBL);

MSBBinary = 8'd99;  LSBBinary = 8'd99;  #20;				// 99.99, mode = 0

$display("%4dns     %b    %2d   %2d | %b %b  %b %b",

$time, ModeSel, MSBBinary, LSBBinary,

HexMSBH, HexMSBL, HexLSBH, HexLSBL);

// mode B (ModeSel=1): count down，show 2:00

ModeSel   = 1;

MSBBinary = 8'd0;   LSBBinary = 8'd0;   #20;				// 2.00, mode = 1

$display("%4dns     %b    %2d   %2d | %b %b  %b %b",

$time, ModeSel, MSBBinary, LSBBinary,

HexMSBH, HexMSBL, HexLSBH, HexLSBL);

// count down 00:05 -> show 01:55

MSBBinary = 8'd0;   LSBBinary = 8'd5;   #20;				// 1.55, mode = 1

$display("%4dns     %b    %2d   %2d | %b %b  %b %b",

$time, ModeSel, MSBBinary, LSBBinary,

HexMSBH, HexMSBL, HexLSBH, HexLSBL);

// count down 02:00 -> show MSB=2,LSB=0

MSBBinary = 8'd2;   LSBBinary = 8'd0;   #20;				// 0.00, mode = 1

$display("%4dns     %b    %2d   %2d | %b %b  %b %b",

$time, ModeSel, MSBBinary, LSBBinary,

HexMSBH, HexMSBL, HexLSBH, HexLSBL);

$display("test finish");

$stop;

end

endmodule

module bin2bcd (

input  wire [7:0] 		bin,    			// binary in 0~255

output wire [3:0] 		tens,   			// bcd out tens

output wire [3:0] 		ones    			// bcd out ones

);

wire [7:0]		bcd_value[0:8];

assign 			bcd_value[0] = 8'd0;		// init

genvar i;

generate

for (i = 0; i < 8; i = i + 1) begin : gen_shift

wire [3:0] cur_tens = bcd_value[i][7:4];

wire [3:0] cur_ones = bcd_value[i][3:0];

// if >=5 + 3

wire [3:0] cor_tens = (cur_tens >= 5) ? (cur_tens + 4'd3) : cur_tens;

wire [3:0] cor_ones = (cur_ones >= 5) ? (cur_ones + 4'd3) : cur_ones;

// result after merging and correcting.

wire [7:0] corrected = { cor_tens, cor_ones };

// Shift left by 1 bit and assign the value of bin[7-i] to the least significant bit.

assign bcd_value[i+1] = { corrected[6:0], bin[7-i] };

end

endgenerate

// assignment of high 4 bits / low 4 bits

assign tens = bcd_value[8][7:4];

assign ones = bcd_value[8][3:0];

endmodule

`timescale 1ns/1ps

module bin2bcd_tb;

reg  [7:0] bin;

wire [3:0] tens, ones;

bin2bcd dut (

.bin(bin),

.tens(tens),

.ones(ones)

);

integer i;

initial begin

$display("time(ns)\t input\t tens\t ones\t result");

// Conduct a traversal test on the range from 0 to 99.

for (i = 0; i < 100; i = i + 1) begin

bin = i;

#10;

if (tens !== i/10 || ones !== i%10) begin

$display("%0t\t%d\t%d\t%d\tFAIL (should be %d,%d)",

$time, bin, tens, ones, i/10, i%10);

end else begin

$display("%0t\t%d\t%d\t%d\tPASS",

$time, bin, tens, ones);

end

end

$display("test finish");

$stop;

end

endmodule

module bcd_to_7seg (

input  wire [3:0] 	bcd,    			// bcd code

output wire [6:0] 	seg     			// 7-seg code

);

// LUT

wire [6:0] seg_show [0:9];

assign seg_show[0] = 7'b1000000;

assign seg_show[1] = 7'b1111001;

assign seg_show[2] = 7'b0100100;

assign seg_show[3] = 7'b0110000;

assign seg_show[4] = 7'b0011001;

assign seg_show[5] = 7'b0010010;

assign seg_show[6] = 7'b0000010;

assign seg_show[7] = 7'b1111000;

assign seg_show[8] = 7'b0000000;

assign seg_show[9] = 7'b0010000;

// output according to the index

assign seg = ( bcd <= 4'd9 )

? seg_show[bcd]

: 7'b1111111;    		// if out of range, all segments will be extinguished.

Endmodule

`timescale 1ns/1ps

module bcd_to_7seg_tb;

reg  [3:0] 	bcd;

wire [6:0] 	seg;

bcd_to_7seg dut (

.bcd	(bcd),

.seg	(seg)

);

initial begin

$display("time(ns)  bcd  ->  seg(abcdefg)");

// test 0

bcd = 4'd0; #10; $display("%0t    %0d  ->  %b", $time, bcd, seg);

// test 1

bcd = 4'd1; #10; $display("%0t    %0d  ->  %b", $time, bcd, seg);

// test 2

bcd = 4'd2; #10; $display("%0t    %0d  ->  %b", $time, bcd, seg);

// test 3

bcd = 4'd3; #10; $display("%0t    %0d  ->  %b", $time, bcd, seg);

// test 4

bcd = 4'd4; #10; $display("%0t    %0d  ->  %b", $time, bcd, seg);

// test 5

bcd = 4'd5; #10; $display("%0t    %0d  ->  %b", $time, bcd, seg);

// test 6

bcd = 4'd6; #10; $display("%0t    %0d  ->  %b", $time, bcd, seg);

// test 7

bcd = 4'd7; #10; $display("%0t    %0d  ->  %b", $time, bcd, seg);

// test 8

bcd = 4'd8; #10; $display("%0t    %0d  ->  %b", $time, bcd, seg);

// test 9

bcd = 4'd9; #10; $display("%0t    %0d  ->  %b", $time, bcd, seg);

// out-of-bound input: Expect seg to be completely wiped out.

bcd = 4'd10; #10; $display("%0t    %0d  ->  %b  (off-limits)", $time, bcd, seg);

$display("test finish");

$stop;

end

endmodule